|  TCR v2.00 Documentation

|

|  Automated Call Return Door

|

|  Copyright 1993,1996 by Tim Strike.

|  Released on December 20th, 1996.

____________

INTRODUCTION

____________

TCR is one of the most versatile call-return doors available on

the market to-date.  The main reason it is so versatile is because

it's a generic door that can be used for several purposes.  TCR

itself does nothing but call the user back, and perform various

security checks and logging . . . everything else is left to batch

files, script languages and external programs, ensuring that TCR

can meet a variety of needs.

TCR can be setup in several different configurations.  This

allows for TCR to be used with ANY BBS software, with any

extensions provided internally or externally by that software.

TCR is based on the Callback Validator type system, except

that TCR does absolutely no "validating".  The concept of TCR was

first established when a few friends were looking for a door that

could be setup to do a return call on certain users.  In some

areas, calls one way are long distance, yet the return call is a

local call. Something like this:

Point1 ------(LD)------> Point2

<---(Local)------

The idea was to get a door going for the Point2 system, where

the Point1 system would start the call, and Point2 would then

callback so that no charges (except the initial dialup) are

incurred by the Point1 system.

So what's the point of the entire thing?  Well, imagine that

Point2 was a BBS, and Point1 was a slew of callers from one

specific area.  Normally, Point2 might find the interest in the BBS

diluted because callers are unwilling to take the costly plunge.

TCR is an attempt at a solution for systems who find themselves in

the Point2 situation with callers in the Point1 area unwilling to

commit.

As an extension to this idea, TCR creates a "success" file. If

the BBS program processes this success file as successful

validation, TCR can be turned into a callback validation system.

The only worries of the various programmers/BBS authors is to

process this file, and validate the user accordingly.  TCR does all

the logistical work of calling the user, keeping logs, etc...

_________________

PROGRAM OPERATION

_________________

TCR requires the DOOR.SYS file to be in the current operating

folder.  If you are running TCR from a different folder, make sure

that somehow the DOOR.SYS file gets copied into the TCR executable

folder or specify the command line /D:<path> which will be used

to load the DOOR.SYS file from that path.

PLEASE NOTE THAT THIS DOOR WILL ONLY WORK AS WELL AS YOU SET

IT UP.  YOU MUST CHANGE THE .DAT FILES.  YOU MUST CHANGE THE MODEM

CONFIGURATION.  And that's likely not all of it.  If you have any

problems getting this door to work, please drop me a line.

_______

SAMPLES

_______

TCR has three data files that is uses to parse/process

numbers. TCR-LOCK.DAT is a file that locks out numbers from the

callback process.  TCR-DIAL.DAT is a file which is used to trim the

phone number from the 1-###-###-### format to something that is

dialable. The TCR-LONG.DAT file contains a list of all the numbers

that you will call back long distance.  If you do not put anything

in this file, even if you have long distance calls set to YES, they

will not be processed.

TCR also uses another data file, TCR-CALL.DAT to store which

users it has called back, their original numbers, names, the dialed

number, password attempts and date/time.  This file has a specific

format which is listed in the sample file; it will change as users

become validated, and their numbers get added to the data file.

If you do NOT have these files, then TCR will operate as

normal; however you will likely end up dialing numbers you don't

want to dial.  The samples are in the file called SAMPLES.ZIP, and

you should UNZIP that file into your TCR folder and read and change

the files.  It is very important that these files be setup for YOUR

area and YOUR setup, as my sample setup files work for my system

only!

____________________

INSTALLATION (BASIC)

____________________

The basic installation is the basis for all the extensions of TCR

(CBV, etc) which are listed later in this document.  Please install

paying specific attention to each step-- DO NOT SKIP A STEP!

1. Create a folder called \fksutil\ on your system

(\bbs\fksutil\ is a suitable folder where \bbs\ is your

main BBS folder).  Create two folders called \tcr\ and

\language\ within the \fksutil\ folder.

\bbs\

} \fksutil\

} \tcr\

\language\

2. Copy *.EXE (TCR, TCRSETUP, MAKELANG) into the \fksutil\tcr\

folder.

3. Unzip SAMPLES.ZIP into the \fksutil\tcr\ folder. Unzip

LANGUAGE.ZIP into the \fksutil\language\ folder.

4. Switch into the \fksutil\tcr\ folder and run TCRSETUP.

Change the defaults and set your modem information. For

more information on the options on these screens, see the

SETUP section of the documentation.

5. Create a batch file to run TCR from your BBS, and place it

in the \fksutil\ folder as TCR.BAT-- unREM the appropriate

line depending on your configuration, and remember to

update the \bbs\ to the appropriate folder.

@ECHO OFF

REM

REM  TCR v2.00 Sample Batch File                              Last Modified:

REM                                                          August 12, 1996

CD \bbs\fksutil\tcr

REM Single node-- Uncomment next line

REM TCR /L:%1 /D:%2

REM Multiple nodes-- Uncomment next line

REM TCR /L:%1 /D:%2 /#:%3

CD \bbs

EXIT

Please note that YOU should restrict access to the door

through your menus.  If you only want people in certain

locations to be called back, create a flag for them and

control the menu access with that flag.

6. In your BBS software, setup a door calling command which

will create DOOR.SYS file, and pass it the following

parameters:

TCR.BAT %l %d %n

where %l is the language filename (if unsupported, hardcode

to ENGLISH), %d is the doorfile path, and %n is the node

number.  Omit the last parameter of using a single node

system.

As an example, the calling sequence in Telegard would be

(although you will need the script file to call the batch

file if you want to do user validation):

TCR.BAT ~VM \bbs\temp\ ~CN      { single node }

TCR.BAT ~VM \bbs\temp~CN\ ~CN   { multiple node }

_______________

"SETUP" COMMAND

_______________

The setup option of TCR can be invoked by simply typing

"TCRSETUP" on the command line.  Once this is done, a screen

similar to the following should appear:

1. Language path   c:\bbs\fksutil\language\

2. Log file        c:\bbs\fksutil\tcr.log

3. # Changes       Yes

4. Boot Callers    Yes

5. Attempts        3

6. Waiting Time    5 seconds

7. Connect Time    30 seconds

8. Random Pass     Yes

9. User Pass       No

0. Pass Attempts   3

A. Lock Previous   No

B. BBS Name        The Call Return Door

L. Long Distance Handling

M. Modem Setup

Option (1) sets the language folder.  This is where you unzipped

the LANGUAGE.ZIP file which contains the language & default text

files.

Option (2) sets the logfile that will be kept by TCR of all the

activities that occur in the door.

Option (3) sets if the user is allowed to input alternate numbers.

If this option is set to NO, the only phone numbers available will

be those given in the DOOR.SYS file -- if those two numbers are the

same, the user will be given one option only.  If YES, the number

the user inputs will be logged.

Option (4) sets the Automated Call Boot.  This option will detect

NO DIAL TONE and RING strings during the callback.  if this option

is set to YES, then the phone will be answered, and a message will

be sent to the connecting user, then TCR will hangup.  This allows

the call return procedure to continue as normal.  IT IS HIGHLY

RECOMMENDED THAT THIS OPTION BE SET TO YES.

Option (5) sets the number of callback attempts that should be

made. If the user doesn't connect in the specified time, TCR will

try again until it has tried this amount.

Option (6) sets the waiting time before the call return starts.

After the user is disconnected, it will pause until this time limit

has been reached, then initialize the modem and start the call

return.

Option (7) sets the waiting time AFTER the number has been dialed.

If no connection is made in this time, the loop will end.  This

should be set to your S7= setting (at minimum).

Option (8) sets if TCR should use a random password to verify that

the user being called back is the user that was on before.

Option (9) sets if TCR should use the user password to verify that

the user being called back is the user that was on before.

Option (0) sets the number of times the user can fail the password.

If the user fails this number of times, they are hungup.

Option (A) sets if TCR should call a number that is has called

before. It is suggested that this option be NO, but for some SysOps

the option has been installed.  If you are going to be using this

door in the call return, not the callback verify mode, then this

option should definately be set to NO.

Option (B) sets the BBS name.  This is displayed to the user when

calling the user back, or when answering the phone in callboot

mode.

Option (L) sets the long distance setup.  When you hit this option, a

screen similar to the following should appear:

1. Allow LD     : No

2. Hangup after : Yes

Morning                 Afternoon

1                   1 1 1                   1 1

2.1.2.3.4.5.6.7.8.9.0.1.2.1.2.3.4.5.6.7.8.9.0.1.

A. Sunday       : XXXXXXXXXXXXXXXXXXXX

B. Monday       : XXXXXXXXXXXXX

D. Wednesday    : XXXXXXXXXXXXX

F. Friday       : XXXXXXXXXXXXXXX

Option (1) sets if the door will allow LD calls.  If this option

is set to NO, any number beginning with 1- will be denied.

Option (2) sets if the door should hangup after the LD call is

completed.  This option, for those of you who are like me, poor,

should be set to YES.  However, if your interface requires that

the user be online to work, using this option isn't possible (in

that case, it is suggested that a menu be created such that the

only exit from the menu is to hangup).

Options (A) through (G) set the times that LD calls are allowed

to be completed.  The days Sunday through Saturday have been

setup with 48 different time slots (every 30 minutes).  If the

timeslot has an X in it, then LD calls are allowed during that

period. This allows you to setup TCR to only dial LD calls when

it is cheapest, etc...

Option (M) [back to the main screen now] sets the modem

configuration.  If you hit this option, a screen similar to the

following will be displayed:

A. Initialization : ATH0M1L0V1E0Q0X6&C1&D2

B. Dialing        : ATDT

C. Answering      : ATA

1. Connect        : CONNECT

2. Busy           : BUSY

3. Error          : ERROR

4. Okay           : OK

5. Ring           : RING

6. Voice          : VOICE

7. No Carrier     : NO CARRIER

8. No Dial Tone   : NO DIAL TONE

Option (A) sets the modem init string.  This should be setup for

your modem to have DROP CARRIER ON DROP OF DTR, and VERBOSE result

strings.  The rest of it you can set according to your modem, but

the above two things must be somehow included in the init string.

The default should work with most modems -- mine's a USR 16.8 DS

and it works great.

Option (B) sets the dialing prefix.  ATDT for Tone, ATDP for

Pulse. Anything beyond that is completely up to you.

Option (C) sets the answering string.  If Call Boot is on, this

will be sent to the modem to answer and boot the caller.

Options (1) through (8) set the modem response strings.  The only

thing of big note is that the CONNECT string should not contain

any of the baud rates, but only a keyword in the string that is

unique to all the "connect" strings of your modem. The defaults

should be fine for most modems.

_____________________

LANGUAGE & TEXT FILES

_____________________

TCR is a completely configurable, multilingual program.  It

can use as little as one language, or as many as you feel like

supporting (limited only by the number of languages the BBS

supports).

To compile the language files, you need to run MAKELANG and

pass it the filename & extra parameters.  This MAKELANG program is

the same language compiler used by the Telegard BBS software, and

thus you must include the proper command lines.  Please review the

file ENGLISH.TX1 in your \fksutil\language\ folder, and then

compile your languages with the command:

\fksutil\makelang.exe \fksutil\language\english.tx1 -f

The language files support displaying of text files instead of

a built-in text line-- for examples, see the default ENGLISH.TX1

file which uses several of the display file lines.  These files use

the extensions .ASC, .ANS and .AVT.

_________________________

SETUP: ONE-WAY LD SYSTEMS

_________________________

The concept behind this setup for TCR is simple, and for the

most part was largely explained during the introduction and initial

setup.  Once you have the "BASIC" setup installed, you can continue

from there.

One-way LD systems occur in remote calling areas which have

"extended" service because they are in a remote area.  These remote

areas have been provided with free calling into the main city, but

the call going the other way FROM the main city is long distance.

This limits the availability of the system to main city callers.

The SysOp of such a system can install a return-dialer (TCR) which

will call the user back.  The call then originates from the remote

area, and is free of cost to the user (save the initial dialup

charge of perhaps 50 cents).

(1) Password checking isn't neccessarily required in this

mode. Since the idea is to setup the door for callers who

are in the main city, calling them back every day, and

having to do double password checks for every logon could

become very annoying.  It is suggested that both the

RANDOM password and USER password checks are turned off.

If your BBS software encrypts the user passwords, user

password checking in TCR *MUST* be turned off.  Random

password checking can still be used.

(2) The lock previous option should also be OFF.  The door

wouldn't work as intended if this option is on, because

the user would be able to use the door once, and then

never again.  Set to NO and leave at NO. ;)

(3) The # Changes field is a matter of personal opinion.  I

personally would choose to put the user's phone number

into the USER RECORD, lock it there, and allow no number

changes in TCR.  This would result in quicker response

time of the system dialing the user back.

There are obviously other things that will need to be checked.  The

text files should be changed accordingly, so that mention of

validation/password checks, etc are removed depending on how the

door has been setup.  Change these files before putting the door

online.

That's more or less it.  Since I don't run this type of system

(live in the main city) I'm not sure of all the intricacies of this

setup. If there are any errors or problems of concern, please feel

free to contact me.

____________________

SETUP: CALLBACK DOOR

____________________

The concept behind this setup is a little different.  First,

it requires more software and a little more work.  However, the

reason that TCR was made into such a portable system was so that it

could easily be moved from BBS to BBS, allowing door authors to

quickly interface TCR to the BBS with minimal effort.

TCR should first be setup in the "BASIC" setup as described in

previous sections of this document.

The next step is EXTREMELY important.  If, and ONLY if the

file SUCCESS.TCR is found, the user should be validated.  There are

several ways of going about this;

(1) The first is to run a script and process this file if it

exists.  This requires a very versatile script language

for the BBS, but there are some powerful BBS script

languages out (Telegard 3.02 supports this feature).

(2) The second is to get a programmer to create a VALIDATION

program, something that will only VALIDATE the user and

do nothing else.  This is simple in itself; the

programmer only need know the record position or user

name, etc to find the neccessary files to change.  Once

this program is written (and there may be some available

for your BBS program already), all you need to do is add

a line to the bach file:

...

CD\BBS\FKSUTIL\TCR

TCR /L:%1 /D:%3 /#:%3

-> IF EXIST SUCCESS.TCR <command>

IF EXIST LONGDIST.TCR <command>

CD\BBS

...

The program can then take over, and validate the user.

The basic concept is that TCR does all the work with the

modem, and this secondary validation program does the

validating. You can understand that 15 BBS programs can

have 15 "validation" programs, and only need one

modem-user interface (TCR).

If there is no validation interface for TCR available

then you might want to look at programming (or finding

someone to program) the interface; since the only thing

involved is maniuplating the data files, it makes it an

easy job.

If someone does program an interface, I'm interested in

carrying the interface on my system so that others can

download it.  Please send/file attach it to me if at all

possible.

(3) If you can think of any other ways, great!  Let me know

so that I can include them.

Once TCR is interfaced to the BBS program, it'll operate without a

problem.  If you find that TCR says that the callback session was

successful, and the user's record doesn't change, the interface

isn't doing it's job -- talk to the author of the interface.

If you are looking for interfaces and can't find one for your

particular BBS system, please drop me a line.  Telegard 3.02 and

subsequent versions all use the built-in scripting language to

validate the users (see \bbs\text\CALLBACK.SCR).  For Renegade

05-11 and compatible, there is a package called TCRRG110.ZIP which

includes the source code.

The process of writing a BBS interface for TCR is extremely easy

and uneventful, because all that is required is a few data file

checks and updates - nothing that requires any great knowledge of

interupts, registers, COM routines, etc - because TCR handles all

of that as the "base" program.

This setup requires a few changes to the BASIC setup as well:

(1) Password checking should DEFINATELY be on for this

callback verifier mode.  One of the reasons that callback

verifications are done are to verify the number AND that

the user online is the user called back.  This can be done

by checking the password.

NOTE: If your BBS software encrypts passwords, the user

password can not be verified.  TURN USER PASSWORD CHECKING

OFF, AND RANDOM PASSWORD CHECKING ON.  This will afford

you the same security (the password is valid for that

session only), and allow you to still properly verify the

user.

The RANDOM password is an option that I highly recommend.

It may annoy a few users who DON'T read the text they are

sent, but that becomes their fault, not yours.  The RANDOM

password is given right before the callback.  It's an 8

character ASCII code (A..Z) which will be different for

every subsequent call (26^8 total possibilities).  This

gives a little more security that the callback is going to

the correct user.

(2) # Changes should be ON.  At least I leave it on, because

some users put their voice number into the user records,

and that's what is contained in the DOOR.SYS, and hence,

what TCR would like to call back.  Allowing number changes

will ensure that the user can be called back at the

correct number (and when a number change is done, a 2 line

reason field is input and stored in the TCR.LOG file).

(3) Callboot should be ON.  This is an option, but it's highly

recommended for a number of reasons.  Incoming calls can

have a number of adverse effects on the callback process.

If ON, the system will answer the phone, tell the user

that a callback is in progress (don't call back for 5

minutes!) and then continue with the original process of

dialing the person back.

And that's more or less it.  The callback verifier/validator option

of TCR is more complex, because of the added interface file which

is required.  However, this provides great flexibility, and it

would be nice if interfaces were developed for the popular BBS

systems on the market today.

____________________

SETUP: CALL SECURITY

____________________

Want to setup communications that will ALWAYS occur in a

specific manner, from a specific destination?  TCR can be setup to

RETURN the call to a specific phone number.  Some might not

understand the entire concept behind this, but it's become popular

where secure sessions are required (terminal programs such as

PcAnywhere provide the service amung others, for those who haven't

setup a BBS).

Setup using the "BASIC" setup, and then make the following

changes:

(1) Number changing should be set to OFF.  This will ensure

that the number that TCR will be forced to callback cannot

be changed.

(2) Random Password checking should be turned OFF.  If the

destination number is forced, there is no point for this

extra security.  Regular password checking should be left

ON.

(3) Lock Previous should be OFF, such that this call-loop can

be done at any given time without being told the process

can't be completed because it's been done once (this is a

multi-session setup in other words, not a one time thing).

Now setup the BBS account to use the number that you wish to

dial back.  For this user account, the first thing that should

occur is a shell to TCR (before getting to the main menu - just

force the logon user into the door).  NOTE: THIS IS ONLY FOR THE

ONE CALLER, OR CALLERS, THAT YOU WISH TO HAVE TCR DO A SECURE DIAL

ON.  SETUP A SPECIAL SECURITY LEVEL IF NECCESSARY.  Now they have

no choice but to go through the door, and no choice but to use the

number in the user record for the dialback.  Now anyone logging on

under that account will be called back at a PRE-defined number,

which means that account can only be used from that PRE-defined

number.

________________________

MULTINODE CONSIDERATIONS

________________________

The largest changes to the 1.10 version of TCR were the

multinode considerations made in the program.  Multinode callback

doors require different options & configurations for different

nodes.  TCR verisons prior to 1.10 didn't take into consideration

these requirements, and anyone installing TCR on a multinode system

would have either run into problems or had to kludge their way to

making it work.

The multinode setup has been outlined in the installation

section of the documentation, including the commands required

to setup the TCR.BAT file for multinode usage.

If you are using the door in CALLBACK VERIFICATION mode, you

must make additional changes to the files you're checking for

success-- SUCCESS.TCR becomes SUCCESS.nnn and LONGDIST.TCR becomes

LONGDIST.nnn where nnn is the node number for the current session.

Lastly, each modem for each node will require a different

setup file.  These files are called MODEM.<node#> and are created

by the TCRSETUP program.  To define your various nodes modem

configurations, run TCRSETUP /#:<node#> for each of the nodes that

you operate on.

__________

LEGALITIES

__________

WARRANTY

TCR is provided as-is, without a warranty of any kind, either

expressed or implied.  It is only guaranteed to occupy disk

space.  Under no circumstances shall the author be liable to you

or anyone else for any damages, including (but not limited to)

any lost profits, lost savings or other incidental or

consequential damages arising out of the use or misuse of this

program.

In english: RUN THIS PROGRAM AT YOUR OWN RISK.  You, and nobody

else is responsible for the outcome of choosing to run this

program.

COPYRIGHT

The program, and all documents and files included in the

original TCR release package are copyrighted by Tim Strike.

They are not to be modified in any way, shape or form.

TCR may be packaged on a distribution diskette or in a

compilation archive, but under no circumstances may any charge

be incurred beyond the physical cost of the hardware (disks, CD,

etc.) or trasfer medium (phone, mail, etc.).

CREDITS

All brand and product names referenced in this document are

trademarks, registered trademarks, or copyrighted works of their

respective holders.

_____________________

CONTACTING THE AUTHOR

_____________________

I can be contacted in a number of different methods.  When

sending netmail or posting in one of the conferences, direct mail

to Tim Strike.  Replies will be forthcoming as soon as I can get to

them.

NETMAIL:

Telenet Canada      Fidonet

20:22/101           1:259/423

INTERNET:

ts@gryn.org

BULLETIN BOARD:

fks Online

905-820-7273

2400-33,600 USR Courier Dual Standard (HST/v34)

Access denied between 4:00am and 5:00am

Restricted access between 9:00pm and 10:00pm

Any changes or upgrades that need to be made to TCR v2.00 will

be made as time permits.  Further suggestions, comments and/or bug

reports can be directed through one of the above methods.

Enjoy fully.